SPECIFICATIONS OF THE SUBJECT

„Acquisition of technologies for large-scale production of disposable medical clothing and PPE aids to the company 2P SERVIS s.r.o. - part 2 - Laminating machine“

| nanofiber producing machine |  |  |  |
| --- | --- | --- | --- |
| Business name of the offered performance: |  | ……………. (to be filled out by the participant) |  |
| Required parametr | Required value |  | Offered value |
| Max Dimensions | 2000 x 2100 x 2100 (H) |  |  |
| Spinning Type | From Bottom to up spinning |  |  |
| Chassis | Electrostatic painted sheet metal |  |  |
| Feeding area material | PE 1000 High Density Chemical resistant |  |  |
| Collector Material | Stainless Steel |  |  |
| Illumination | LED |  |  |
| Total weight | Less than 650 Kgs. |  |  |
| High Voltage Power Supplies | 4 HVPS (Independantly adjustable) Min. 40kV 0.1 KV Precision Min. 4 nozzle rods with multiple nozzles Each set of nozzles is connected to an independant HVPS |  |  |
| Syringe Pumps. | 4 Syringe Pumps Flow rate 0,01 - 4464 ml/h Precision 0,01 ml Suitable with all commercial syringes Each set of nozzles is connected to an independant Syringe Pump Possibility to work with up to 4 different polymers |  |  |
| Minimum number of nobles | 56 |  |  |
| Minimum number of feeding pipes | 4 |  |  |
| Minimum number of nozzles in each pipe | 14 |  |  |
| Possibility to work with 1 nozzle | Yes |  |  |
| Feeding pipe material | Aluminium |  |  |
| Nozzle Material | Electrically Conductive Stainless Steel |  |  |
| Nozzle Inner Diameter | 0,8mm |  |  |
| Min. required solution for Single Nozzle Feeding | 3ml |  |  |
| Min. required solution for each feeding pipe set | 15ml |  |  |
| Min. required solution for full loading | 50ml |  |  |
| Spinning distance | 30 - 280mm |  |  |
| Spining distance adjustment | Automatic Adjustement |  |  |
| Distance adjustment precision | 1 mm |  |  |
| Collector type | Roll to roll collector |  |  |
| Minimum Fiber Deposition Width | 550mm |  |  |
| Substrate Winding Speed | 0,1m/min - 10 m/min |  |  |
| Coating Homegeneity System | X-axis repetitive motion |  |  |
| Stroke of Coating Homogeneity system | 20 mm - 80 mm |  |  |
| Speed of Coating Homogeneity system | 5 - 50 mm/sec |  |  |
| User Interface. | 12" Touch Screen control panel Fully able to control all parameters Saving & Recalling all parameters |  |  |
| Safety functions. | Safety door Emergency Button & Safety Relay Warning Light Electrical Isolated & Grounded Cabinet Over current Protection |  |  |
| Technical requirements. | 110 - 120 VAC or 220 - 240 VAC Max 25A |  |  |
| Heating of spinning area. | Up to 40 °C. |  |  |
| Humidity adjustment of spinning area | Between 25% and room condition |  |  |