Table Specification of the subject of performance

for a contract called:

"Water savings in the company VETROPACK MORAVIA GLASS, akciová společnost"

The contracting authority specifies for the suppliers special technical conditions for the subject of the contract, which are defined in the document "Table of specifications of the subject of performance". The contracting authority defines in the technical conditions the characteristics of the requested subject of performance, which must be met by the offered subject of performance of the suppliers. The supplier shall indicate in the Technical Conditions whether the performance offered by it meets the requirements specified in the columns by selecting the appropriate option in the "Meets" column, "Yes" if the performance offered meets this requirement and "No" if the performance offered does not meet this requirement. In case the supplier indicates "No" at least once in the technical conditions, it will be excluded for non-compliance. In the event that a supplier indicates "Yes" and the evaluation of the tenders reveals that the goods offered do not meet this requirement, the supplier may be excluded for non-compliance and breach of the tender conditions. In the event that a supplier does not complete either the "Yes" or "No" option, the supplier may be excluded for failure to meet the terms and conditions. In the "Supplier offers" column, the supplier shall then indicate the specific value of the parameter (in the same units as the requirement) or a more detailed specification of the performance offered in relation to the requirement. In case the supplier does not fill in the column "Supplier offers" and in the column "Meets" ticks the option "Yes", it is considered that the supplier's offered performance exactly corresponds to the requirement of the contracting authority, set out in the columns "Required parameter" and "Required value".

Supplier will supply 1pc of new and unused washer liner, as specified below.

| The trade name of the performance offered by the participant: | (the participant shall add the trade name of the machine offered) |  |  |
| --- | --- | --- | --- |
| Required parameter | Required value | Meets | The supplier offers |
| Washing line for polypropylene liners in dimensions (mm) Dimensional tolerance min. ± 5 %. | PP-PLATE 1000/1200/3.0 2250 G/M2 FULL. PP-PLATE 800/1200/3.5 1000 G/M2. PP-PLATE 1000/1200/3.5 1000 G/M2. PP layer pad 1200x1000 radius 100, green. PP-PLATE 800/1200/3.0 2250 G/M2 FULL. RP POLIPR.1210X1010mm C.CHIUSE ( 1000 G/ M2). PP-PLATE 1000/1200/3.5 1000 G/M2 RED; radius 60. | YES/NO |  |
| Capacity of the washing line | Min. 1250 pcs/hr. | YES/NO |  |
| Packaging (inlet/outlet of the washing line) - pallets with dimensions (l x w x h), (mm). Dimensional tolerance ± 5%. | 1.200 x 800 x 150 1.200 x 1.000 x 150 1.210 x 1.010 x 150 | YES/NO |  |
| Permitted pack height (all parts of the washing line) | 200 mm - 1200 mm | YES/NO |  |
| Permissible pack weight (inlet/outlet of the washing line) | 30 kg - 900 kg | YES/NO |  |
| Operating hours | 3 shifts, 8 hour shift min. 360 days per year | YES/NO |  |
| Placement in the existing space of 30 x 24 m | Preservation of the existing storage and handling space according to the drawing - PDF attachment | YES/NO |  |
| Required energy | Electrical power 3x400 V 50 Hz | YES/NO |  |
| Required energy | Compressed air - adapt to existing compressor 7 bar | YES/NO |  |
| Required energy | Drinking water - adapt to existing 6 bar supply | YES/NO |  |
| Parts of the washing line | Sorting, washing, packing | YES/NO |  |
| Sorting part | Connected by automatic transport routes with washing parts | YES/NO |  |
| Packing part | Automatic packing of a defined number of pieces on a pallet | YES/NO |  |
| Packing part | Machine wrapping of films, wrapping part automatically connected to the washing part | YES/NO |  |
| Maximum water consumption | Max. 1080 L/shift | YES/NO |  |
| Sterilisation at the outlet | Below 95 RLU | YES/NO |  |
| Strip counters | at input and output + discarded | YES/NO |  |
| Documentation (operating instructions, technical specifications,...) | in Czech language | YES/NO |  |
| Documentation requirements | Documentation including P&I Diagram, address location of all sensors with function description In wiring diagrams, use component labelling by page number and position on the page. The documentation will include: schematics, spare parts list, lubrication schedule, planned maintenance Declaration of conformity, initial electrical inspection. | YES/NO |  |
| Line status diagnostics | Min. current state, fault states, global system reset. Operator panels on each machine, visualisation in Czech language, display of operating status of all devices and drives. Addressable fault messages and fault archive. | YES/NO |  |
| Commissioning training | Operation + maintenance | YES/NO |  |
| Warranty period | 2 years | YES/NO |  |
| Service support | min. 10 years, support - 8/5 + remote access + service support | YES/NO |  |
| Availability of spare parts | Min. 10 years | YES/NO |  |
| Possibility of service control | Via HMI - operator panel or controller | YES/NO |  |
| Detection of bonded inserts at the input | Including spacer separation system | YES/NO YES/NO |  |
| Number of persons per operator/shift | 2 persons | YES/NO |  |
| Delivery conditions | DAP Kyjov, plot 436/78, cadastral area Kyjov [678431] | YES/NO |  |
| Maturity of the invoice | Min. 60 days | YES/NO |  |
| Parameters that are subject to evaluation of tenders: |  |  |  |
| Energy consumption | To be completed by the participant |  |  |
| Water consumption | To be completed by the participant |  |  |
| Washing capacity | Min. 1250 pcs/hr. |  |  |
| Total line noise | 75 dB - 85 dB |  |  |

I (we) the undersigned (i)       do solemnly declare that the above information is true and that the supplier       if selected by the contracting authority in the subject tender, will deliver the goods exactly in accordance with the technical and commercial conditions in its tender.

In     on

_________________________________

Name and function of the authorised person of the supplier

Stamp and signature of an authorised person of the supplier